Flexibility factor KPIs

- Nomenclature:

| Pt | Hourly energy price at time interval t |
| --- | --- |
| qESS,t | Hourly ESS discharge energy at time interval t |
| qh,t | Hourly energy consumption at time interval t |
| qGen,t | Hourly generated energy at time interval t |
| lpt/hpt | Low/high price time |
| q∆,t | Hourly the net energy balance at time interval t |
| CostqGen | Total energy cost of generated energy |
| Costqh | Total energy cost of energy consumption |
| CostqESS | Total energy cost of ESS discharge energy |
| PC | Procurement cost |

Flexibility Factor:

Literature

Ability to shift the energy use during time with high prices to low energy price periods

(S) It explains how the energy demand is distributed in comparison to the energy peaks.

(W) It doesn’t give any further information on how much local load can be shifted.

-1≤FF=lptqhdt-hptqhdtlptqhdt+hptqhdt≤1	(1)

- Gives a quick indication of when heating energy is consumed

- FF = 0 if demand is similar during both price periods

- FF = 1 (max) or -1 (min), if demand in single pricing period

qh : Heating demand

The flexibility factor (FF, Equation (20)) is defined as the ability to shift energy use from periods with high energy prices to periods with low energy prices [25]. It gives an indication of when energy is consumed. In particular, if no demand load occurs during low price time its value is -1, if no demand load occurs during high price time its value is 1 and if demand load is similar during both price time its value is 0. It can be used to quantify the energy can be shifted in terms of heat stored by thermal mass.

Questions:

- What about the normal price time?

- We might have lpt with different prices the same as hpt. Is it different if we shift all the consumption to the lowest price or just distribute it among all price levels? (Cost is not considered)

- How do we categorize lpt and hpt? How the prices of lpt and hpt change?

- What is the time frame for calculating FF? Yearly, daily or average basis?

- How do we calculate representative days, prices and values?

- What about ESS and generation effects?

Point:

- The point of this factor is to define the amount of flexibility of demand to be shifted

- We need to add generation and ESS effects to this formula

Solutions:

Assumptions:

- Time frame:

  - Daily, for some representative days

- Prices higher than median: hpt

- Prices lower than median: lpt

- qESS,t≤0, charge

- qESS,t≥0, discharge

- qGen,t≥0, Generated power

- qGen,t≤0, Imported power

- q∆,t= qh,t-qESS,t-qGen,t

New formulations:

- Adding hourly price Pt to the main FF (1)

FFqh=lptPt.qh,tdt-hptPt.qh,tdtlptPt.qh,tdt+hptPt.qh,tdt	(2)

- Replacing consumption with net energy balance to consider impacts of generation, consumption and ESS

FFq∆=lptPt.q∆,tdt-hptPt.q∆,tdtlptPt.q∆,tdt+hptPt.q∆,tdt	(3-a)

FFq∆=lptPt.qh,tdt-hptPt.qh,tdtlptPt.q∆,tdt+hptPt.q∆,tdt-lptPt.qESS,tdt-hptPt.qESS,tdtlptPt.q∆,tdt+hptPt.q∆,tdt-lptPt.qGen,tdt-hptPt.qGen,tdtlptPt.q∆,tdt+hptPt.q∆,tdt=FFqhlptPt.qh,tdt+hptPt.qh,tdtlptPt.q∆,tdt+hptPt.q∆,tdt-FFqESSlptPt.qESS,tdt+hptPt.qESS,tdtlptPt.q∆,tdt+hptPt.q∆,tdt-FFqGenlptPt.qGen,tdt+hptPt.qGen,tdtlptPt.q∆,tdt+hptPt.q∆,tdt= FFqhCostqh-FFqESSCostqESS-FFqGenCostqGenlptPt.q∆,tdt+hptPt.q∆,tdt  	(3-b)

- Formulating FF_shift for net energy balance cost

FFShift, q∆=FF q∆-FF q∆,refFF q∆,ref	(4)

- Formulating contribution of generation, consumption and ESS separately to the FF.

FFcontribution,qh=lptPt.qh,tdt-hptPt.qh,tdtlptPt.q∆,tdt+hptPt.q∆,tdt=FFqhCostqhlptPt.q∆,tdt+hptPt.q∆,tdt	(5-a)

FFcontribution,qESS=lptPt.qESS,tdt-hptPt.qESS,tdtlptPt.q∆,tdt+hptPt.q∆,tdt=-FFqESSCostqESSlptPt.q∆,tdt+hptPt.q∆,tdt	(5-b)

FFcontribution,qGen=lptPt.qGen,tdt-hptPt.qGen,tdtlptPt.q∆,tdt+hptPt.q∆,tdt=-FFqGenCostqGenlptPt.q∆,tdt+hptPt.q∆,tdt	(5-c)

Flexibility factor FFPC (costs)

Literature

In terms of procurement costs (PC) avoided.

(S) It takes into account the operational cost savings.

(S) Although, the authors use this KPI to evaluate the ability to shift the heat pump electric load, it can be used to investigate the flexibility of any other electrical equipment.

- Annual PC varies due to electricity time of use (ToU) tariffs.

- FF maximizes as PC → PCmin if all heating is done during cheapest ToU.

FFPC=PCmax-PCPCmax-PCmin	(6)

PC : Procurement cost of the electricity consumed per year

The procurements cost avoided flexibility factor (FFPC, Equation (24)) is used to quantify the building flexibility in terms of procurement costs avoided (cost savings) [66]. The minimum value 0 is reached when the electricity required is used at the time with the highest price while the maximum value 1 is achieved at the time with the lowest price. Although the authors use this KPI to evaluate the ability to shift the heat pump electric load, it can be used to investigate the flexibility of any other electrical equipment.

Questions:

            - How should we calculate maximum and minimum costs? How is it possible to calculate the minimum cost for a year of operation?

            - How should the consumption be distributed in lpt time intervals when we have different price values in lpt period?

            - How long is the lpt time frame? Is it calculated yearly or daily?

            - How could we consider ESS and generation impacts?

Solutions:

Assumptions:

New formulations:

Flexibility factor FFShift (volume)

Literature

In terms of energy shifted compared to a reference profile.

Ability to shift the heat pump electric load from peak to off-peak hours in terms of energy shifted compared to a reference profile

- FFPC,ref: flexibility in terms of PC for a flat tariff reference case

(S) It can be used to investigate the flexibility of any other electrical equipment.

FFVS=FFPC-FFPC,refFFPC,ref

=PCmax-PCPCmax-PCmin-PCmax-PCrefPCmax-PCminPCmax-PCrefPCmax-PCmin=PCref-PCPCmax-PCref	(7)

The volume shifted flexibility factor (FFVS, Equation (25)) is used to account for the flexibility in terms of energy shifted compared to a reference profile [66]. It can be used to investigate the flexibility of any other electrical equipment.

Questions:

Solutions:

Assumptions:

New formulations:

- Formulating FF_shift for net energy balance cost (mentioned earlier)

FFShift, q∆=FF q∆-FF q∆,refFF q∆,ref	(4)